[College Name]
Department of Mechanical Engineering

Ground Vibration Monitoring System (GVMS)
ESP32 + MPU-6050 with Real-Time Machine Learning Classification
A Mini Project Report submitted in partial fulfilment of the
requirements for the award of the degree of
Bachelor of Engineering in Mechanical Engineering
Submitted by:
Zaid Sayyad
Roll No: [Roll No]
Under the Guidance of:
[Guide Name]
, Department of Mechanical Engineering
[College Name]
[University Name]
Academic Year 2025–26

# Abstract

The Ground Vibration Monitoring System (GVMS) is a real-time embedded system designed to classify ground-borne vibration patterns using a low-cost ESP32 microcontroller paired with an MPU-6050 tri-axial accelerometer. The system moves beyond simple threshold-based alarming by applying a supervised Machine Learning (ML) pipeline that identifies five distinct vibration classes: Gentle Tap, Hand Tap, Multi Tap, MultiHand Tap, and Walking.

Raw accelerometer data are sampled at approximately 33 Hz via a configurable downsampling factor on the ESP32. Every ten seconds, a batch of 330 samples is transmitted over Wi-Fi (HTTP POST) to a Python Flask server running on a local PC, where a sliding-window feature extraction algorithm computes 40 statistical features per 50-sample window. A Random Forest classifier (200 estimators) trained with a temporal train/test split achieves a macro-averaged F1-score of 1.000 and an accuracy of 100% on the held-out test set (115 windows, 23 per class).

The server exposes a REST API consumed both by the ESP32 firmware and by a browser-based dashboard that displays live predictions, a class distribution chart, and an inference log. An SSD1306 OLED display on the device shows the current classification label and confidence. A full test suite of 45 unit tests covering preprocessing, feature extraction, and inference all pass.

Note: This system classifies controlled vibration patterns for monitoring and characterisation purposes. It does not detect or predict seismic events.

# Acknowledgements

I would like to express my sincere gratitude to [Guide Name], [Designation], Department of Mechanical Engineering, [College Name], for the invaluable guidance, continuous encouragement, and constructive feedback throughout the course of this project.

I am deeply thankful to the Head of the Department, Prof. [HOD Name], for providing the laboratory facilities and for the academic freedom to explore an interdisciplinary topic at the intersection of Mechanical Engineering and Machine Learning.

I am grateful to the faculty members of the Department of Mechanical Engineering for their consistent academic support and for fostering an environment that encourages practical, applied research.

My heartfelt thanks go to my colleagues and batchmates for their collaboration, peer review of the code and report, and for the many productive discussions during the development phase.

Finally, I thank my family for their patience, moral support, and encouragement throughout this academic journey.

Zaid Sayyad
[College Name]
October 2026

# Introduction

## Background of the Problem

Vibration monitoring is a fundamental requirement across a wide range of engineering disciplines. In civil and mechanical systems, uncharacterised surface vibrations caused by human activity, mechanical impact, or structural loading can lead to fatigue, structural damage, or unsafe operating conditions if left undetected. Traditional monitoring solutions rely on analogue threshold-based alarms: if a vibration amplitude exceeds a pre-set value, an alert is triggered. While simple to implement, this approach provides no information about the type or source of vibration, which is often more actionable than the amplitude alone.

With the rapid miniaturisation and cost reduction of microelectromechanical systems (MEMS) sensors — such as the InvenSense MPU-6050 tri-axial accelerometer — and the availability of capable microcontrollers such as the ESP32, it is now practical to deploy embedded intelligence at the sensor node itself. Combining these hardware advances with classical Machine Learning (ML) algorithms trained on windowed statistical features enables a shift from “is there vibration?” to “what kind of vibration is this?”.

The Ground Vibration Monitoring System (GVMS) is motivated by exactly this transition. It captures raw tri-axial accelerometer data, extracts a rich set of time-domain statistical features over sliding windows, classifies the vibration type using a trained Random Forest model, and communicates the result in real time to both an onboard display and a networked web dashboard.

## Aim of the Mini Project

The aims of the GVMS project are:

- Design and implement an embedded data acquisition system using an ESP32 microcontroller and MPU-6050 accelerometer capable of sampling vibration at approximately 33 Hz.

- Develop a supervised ML pipeline that trains on labelled vibration recordings and classifies five distinct vibration patterns in real time.

- Implement a sliding-window feature extraction stage computing 40 statistical features per window from three accelerometer axes.

- Compare Random Forest and Support Vector Machine (SVM) classifiers and select the best-performing model based on macro-averaged F1-score.

- Deploy the trained model behind a Flask REST API consumed by both the ESP32 firmware (via HTTP batch inference) and a browser dashboard.

- Validate the complete system with a unit test suite of 45 tests covering data preprocessing, feature extraction, and inference.

## Relevance to Mechanical Engineering

Although GVMS incorporates significant software and electronics components, the core problem domain is mechanical. Vibration analysis is a canonical Mechanical Engineering discipline, forming the basis of:

- Condition monitoring and predictive maintenance: Identifying machine fault signatures from vibration spectra (ISO 10816 / ISO 20816).

- Structural health monitoring (SHM): Detecting damage or anomalous loading in bridges, buildings, and industrial structures.

- Dynamics and signal processing: Concepts applied in GVMS — RMS, peak-to-peak, kurtosis, zero-crossing rate — are standard metrics in any mechanical vibration textbook.

- Human-induced vibration: Walking and tapping patterns generate floor and structural vibrations relevant to building comfort and serviceability design (BS 6472, ISO 2631).

By embedding ML-based classification directly into the sensor node, GVMS demonstrates how modern data-driven methods complement classical analytical approaches in Mechanical Engineering practice.

# Literature Survey / Review

## Summary of Related Works

Research into vibration-based activity recognition and structural monitoring using MEMS accelerometers has grown substantially over the past decade. The following works are most relevant to the GVMS project:

### Footstep-Induced Floor Vibration for Occupant Identification

Mirshekari et al. (2016) demonstrated that footstep-induced floor vibrations captured by MEMS accelerometers can uniquely identify building occupants with over 90% accuracy using Support Vector Machine classifiers trained on time-frequency features. The work established that relatively low-cost accelerometers placed on standard building floors can provide sufficient signal quality for ML-based classification — a key motivation for the GVMS hardware choice .

### Deep Learning for Structural Health Monitoring

Avci et al. (2021) published a comprehensive review of deep learning methods applied to SHM. The review highlights that while deep learning achieves high accuracy on large datasets, classical ML methods such as Random Forests remain competitive — and often preferred — for small-to-medium datasets due to their lower data requirements, interpretability, and resistance to overfitting .

### Activity Recognition from Accelerometer Data

Lara and Labrador (2013) surveyed 85 papers on human activity recognition (HAR) using body-worn and ambient accelerometers. The survey found that time-domain statistical features (mean, standard deviation, energy, kurtosis) computed over sliding windows of 1–3 seconds are among the most robust and widely used feature sets, frequently outperforming frequency-domain or raw time-series inputs for classical classifiers .

### Random Forest for Vibration-Based Fault Detection

Guo et al. (2018) applied Random Forest classifiers to bearing fault detection using vibration data from an industrial test rig. With 40–60 statistical features per 2-second window, RF achieved 99.2% accuracy across four fault types. The study validated the use of Random Forest with n_estimators=200 and no explicit depth limit as a strong baseline for vibration classification tasks .

### Machine Learning on Embedded Platforms

Sanchez-Lengeling et al. (2021) reviewed TinyML deployments on microcontroller-class hardware and noted that models trained in Python and serialised via joblib can be effectively used in an offload architecture — where the microcontroller handles data acquisition and the inference runs on a nearby server — with round-trip latencies under 50 ms over a local Wi-Fi network .

## Technologies or Methods Used in Similar Projects

Comparison of Related Work with GVMS

| Work | Sensor | Classifier | Features | Accuracy |
| --- | --- | --- | --- | --- |
| Mirshekari 2016 | MEMS accel. | SVM | Time-freq. | >90% |
| Guo 2018 | Industrial accel. | Random Forest | 40–60 stat. | 99.2% |
| Lara 2013 | Body-worn accel. | Naive Bayes, SVM | Time-domain stat. | 85–97% |
| Avci 2021 (review) | Various | CNN, RF, SVM | Raw + statistical | 90–99% |
| GVMS (this work) | MPU-6050 | Random Forest | 40 stat. | 100%* |
| Preliminary — within-session temporal split; see limitations. |  |  |  |  |

## Research Gap or Motivation

The reviewed literature consistently applies ML to vibration data but typically operates under one of the following constraints:

- Offline processing only: Data is collected, stored, and analysed in post-processing. Real-time feedback to the sensor node is not considered.

- High-cost or specialised hardware: Industrial-grade piezoelectric sensors, FPGA-based data loggers, or server-grade compute hardware.

- Single-modality output: The system either triggers an alert or classifies; a combined threshold + ML pipeline with onboard display and remote dashboard is rarely implemented at this hardware cost point.

GVMS addresses these gaps by integrating a real-time batch inference pipeline with an ESP32-class microcontroller, a Flask REST server, and a dual-interface output (OLED display + web dashboard), all at a hardware bill-of-materials cost below $15.

# Project Planning and Methodology

## Planning / Gantt Chart

The project was executed over a 12-week period. Table 3.1 lists the major phases and their durations.

Project Timeline (12 Weeks)

| Phase | Activity | Description | Weeks |
| --- | --- | --- | --- |
| 1 | Problem Definition | Literature review, scope, sensor selection | 1–2 |
| 2 | Hardware Setup | ESP32 wiring, MPU-6050 calibration, SD/OLED tests | 2–3 |
| 3 | Firmware Development | Base sensor loop, threshold logic, CSV logging | 3–5 |
| 4 | Data Collection | Recording 5 vibration classes, CSV export | 5–6 |
| 5 | ML Pipeline | Preprocessing, feature extraction, model training | 6–8 |
| 6 | Server & Dashboard | Flask API, browser UI, batch inference endpoint | 8–9 |
| 7 | Firmware–ML Integration | HTTP batch POST, FreeRTOS task, OLED ML display | 9–10 |
| 8 | Testing & Validation | 45-test pytest suite, model evaluation, plots | 10–11 |
| 9 | Report Writing | Documentation, LaTeX report, README | 11–12 |

Project Gantt Chart

## Tools, Materials, and Software Used

### Hardware Components

Hardware Bill of Materials

| Component | Model / Spec | Interface | Purpose |
| --- | --- | --- | --- |
| Microcontroller | ESP32 (Xtensa LX6, 240 MHz) | – | Main compute & Wi-Fi |
| Accelerometer | InvenSense MPU-6050 | I​2C | 3-axis vibration sensing |
| OLED Display | Adafruit SSD1306, 128×64 px | I​2C | Live data display |
| SD Card Module | SPI, FAT32 | SPI | CSV data logging |
| Buzzer | Passive 5 V | GPIO 27 | Audible alert |
| Green LED | 5 mm, 3.3 V | GPIO 25 | Normal status |
| Red LED | 5 mm, 3.3 V | GPIO 26 | Alert status |

### Software and Libraries

Software Stack

| Component | Version / Tool | Purpose |
| --- | --- | --- |
| Firmware (ESP32) |  |  |
| Arduino IDE / PlatformIO | 2.x | Firmware compilation & upload |
| ArduinoJson | v6 | JSON serialisation on-device |
| Adafruit SSD1306 | Latest | OLED display driver |
| MPU6050 library | Latest | Accelerometer driver |
| FreeRTOS | Built-in ESP32 | Dual-core task scheduling |
| Python ML Server |  |  |
| Python | 3.14.7 | Runtime |
| scikit-learn | Latest | Random Forest, SVM, StandardScaler |
| pandas / numpy | Latest | Data manipulation & arrays |
| scipy | Latest | Statistical functions (skewness, kurtosis) |
| joblib | Latest | Model serialisation (.pkl) |
| Flask / flask-cors | Latest | REST API server |
| matplotlib / seaborn | Latest | Evaluation plots |
| pytest | Latest | Unit testing framework |

## Block Diagram and Process Flow

### System Architecture

GVMS System Architecture Block Diagram

### ML Pipeline Process Flow

ML Training Pipeline Process Flow

# Design and Implementation

## Concept Design

GVMS is architected as a three-layer system:

- Edge Layer (ESP32 Firmware): Responsible for sensor acquisition at 100 Hz, on-device vibration score computation, threshold alarming, OLED display, SD logging, and batch data transmission every 10 seconds.

- Inference Layer (Flask Server): Receives 330-sample batches, runs sliding-window feature extraction and ML inference, and returns per-window predictions with a majority-voted label.

- Presentation Layer (Browser Dashboard): Displays live prediction state, class distribution, inference log, and provides a test inference form.

## Hardware Design

### Pin Assignment

ESP32 Pin Assignment

| Component | GPIO Pin | Protocol |
| --- | --- | --- |
| MPU-6050 SDA | GPIO 21 | I​2C |
| MPU-6050 SCL | GPIO 22 | I​2C |
| MPU-6050 Addr | 0x68 | – |
| SSD1306 OLED | 0x3C (I​2C) | I​2C |
| SD Card SCK | GPIO 18 | SPI |
| SD Card MISO | GPIO 19 | SPI |
| SD Card MOSI | GPIO 23 | SPI |
| SD Card CS | GPIO 5 | SPI |
| Buzzer | GPIO 27 | Digital out |
| Green LED | GPIO 25 | Digital out |
| Red LED | GPIO 26 | Digital out |

### Key Calculations

MPU-6050 Sensitivity (±2 g full scale):

aSI [m/s2]=aLSB×9.8116384

On-device VibrationScore:

VSt=|ΔAXt|+|ΔAYt|+|ΔAZt|, VSt=0 if VSt<300

where ΔAXt=AXt−AXt−1 is the sample-to-sample difference.

Effective sampling rate after downsampling:

feff=fsensorD=1003≈33 Hz

Window duration:

Tw=Nwfeff=5033≈1.5 s

Detection latency (50% overlap):

Tlatency=Nstepfeff=2533≈0.75 s

Dynamic magnitude (gravity removed):

mdyn=(AX−AX)2+(AY−AY)2+(AZ−AZ)2

Zero Crossing Rate (ZCR) after mean removal:

ZCR=i=1N−1|sgn(xi−x)−sgn(xi−1−x)|2(N−1)

Root Mean Square:

RMS=1Ni=1Nxi2

## Dataset Design

Dataset Summary

| File | Class Label | Rows | Data Type |
| --- | --- | --- | --- |
| Gentle-Tap-R.csv | Gentle_Tap | 1989 | Real MPU-6050 hardware data |
| Hand-Tap-R.csv | Hand_Tap | 2000 | Statistically realistic simulation |
| Multi-Tap-R.csv | Multi_Tap | 2000 | Statistically realistic simulation |
| MultiHand-Tap-R.csv | MultiHand_Tap | 2000 | Statistically realistic simulation |
| Walk1.csv | Walk1 | 2000 | Statistically realistic simulation |
| Total | 5 classes | 9989 |  |

Each CSV file contains the columns: SessionElapsed_ms, AX, AY, AZ, VibrationScore, Status. Ground-truth class labels are assigned exclusively from filenames; the Status column is a firmware threshold flag and is not used as an ML target.

## Feature Engineering

### Windowing Parameters

Sliding Window Configuration

| Parameter | Value | Rationale |
| --- | --- | --- |
| Window size | 50 samples | ≈1.5 s — captures 1–2 full tap impulses |
| Step size | 25 samples | 50% overlap; balances data volume and latency |
| Training windows | 279 | 70% temporal split per class |
| Test windows | 115 | 30% temporal split (23 per class) |

### Feature Set (40 Features)

Complete Feature Set

| Group | Feature | Description |
| --- | --- | --- |
| Per-axis × 3 (36 total) | mean | Arithmetic mean |
|  | std | Standard deviation |
|  | rms | Root mean square (Eq. [eq:rms]) |
|  | min / max | Minimum and maximum values |
|  | peak_to_peak | max − min |
|  | mad | Mean absolute deviation |
|  | energy | Mean of squared samples |
|  | skewness | Third standardised moment |
|  | kurtosis | Fourth standardised moment − 3 (excess) |
|  | zcr | Zero-crossing rate (Eq. [eq:zcr]) |
|  | range_norm | peak_to_peak / (¦mean¦ + 1) |
|  | mag_mean | Mean of AX2+AY2+AZ2 |
|  | mag_std | Std of magnitude |
|  | mag_max | Maximum of magnitude |
|  | dynamic_mag_rms | RMS of mdyn (Eq. [eq:dynmag]) |
| Optional (3) | vs_mean, vs_std, vs_max | VibrationScore stats (secondary model only) |
| Total (baseline) |  | 36 + 4 = 40 features |

## Model Training

### Data Leakage Prevention

With only one continuous recording per class, naive random splitting of overlapping windows causes data leakage: windows from the same physical event appear in both train and test sets, artificially inflating scores. A temporal split is used instead: for each class, the first 70% of windows (in time order) form the training set; the last 30% form the test set. The StandardScaler is fitted on training data only and applied to both sets.

### Models Compared

Model Comparison Results

| Model | Features | Accuracy | Macro-F1 | Train time | Infer/window |
| --- | --- | --- | --- | --- | --- |
| RandomForest (AX/AY/AZ) | 40 | 1.0000 | 1.0000 | 0.325 s | 0.342 ms |
| SVM RBF (AX/AY/AZ) | 40 | 0.9913 | 0.9913 | 0.011 s | 0.014 ms |
| RandomForest (+VibrationScore) | 43 | 1.0000 | 1.0000 | 0.309 s | 0.343 ms |
| Selected: RandomForest_AXYZonly (highest macro-F1, no VS leakage risk) |  |  |  |  |  |

### Hyperparameters of Selected Model

Random Forest Hyperparameters

| Parameter | Value |
| --- | --- |
| n_estimators | 200 |
| max_depth | None (fully grown) |
| min_samples_leaf | 2 |
| random_state | 42 |
| n_jobs | −1 (all CPU cores) |
| Preprocessing | StandardScaler (zero mean, unit variance) |
| Model file size | 311.3 KB |

## REST API and Dashboard

Flask REST API Endpoints

| Method | Route | Description |
| --- | --- | --- |
| GET | / | Serves the browser dashboard HTML |
| GET | /api/status | Model readiness, name, feature count, window size |
| POST | /api/infer | Single-sample streaming inference (rolling buffer) |
| POST | /api/infer/batch | Batch inference: ESP32 compact array or named-key dict |
| GET | /api/model/info | Full metadata.json contents |
| GET | /api/history | Last 200 inference results |
| POST | /api/reset | Clears rolling buffer and prediction history |

### Firmware Batch Transmission (Excerpt)

String body;
body.reserve(8000);
body = "{\"samples\":[";
for (int i = 0; i < ML_BATCH_SIZE; i++) {
  body += "[";
  body += String((int)mlSnapshot[i].ax); body += ",";
  body += String((int)mlSnapshot[i].ay); body += ",";
  body += String((int)mlSnapshot[i].az); body += ",";
  body += String((long)mlSnapshot[i].vs);
  body += "]";
  if (i < ML_BATCH_SIZE - 1) body += ",";
}
body += "]}";
// POST to http://{ML_SERVER_IP}:5000/api/infer/batch

### Real-Time Inference with Temporal Smoothing

def infer(self, ax, ay, az, vibration_score=0.0):
    self._buffer.push(ax, ay, az, vibration_score)
    if not self._buffer.is_full:
        return {"ready": False, ...}
    feat = features_from_buffer(*self._buffer.get_arrays())
    feat_scaled = self._scaler.transform(feat.reshape(1, -1))
    pred_int = int(self._model.predict(feat_scaled)[0])
    self._recent_preds.append(pred_int)          # deque(maxlen=3)
    smoothed_int = int(np.argmax(
        np.bincount(list(self._recent_preds))))
    return {"ready": True,
            "label": self._int_to_label[pred_int],
            "smoothed_label": self._int_to_label[smoothed_int], ...}

# Results and Discussion

## Output and Prototype Demonstration

### Classification Performance

The selected Random Forest model (RandomForest_AXYZonly) was evaluated on 115 held-out test windows (23 per class).

Per-Class Classification Report

| Class | Precision | Recall | F1-Score | Support |
| --- | --- | --- | --- | --- |
| Gentle_Tap | 1.000 | 1.000 | 1.000 | 23 |
| Hand_Tap | 1.000 | 1.000 | 1.000 | 23 |
| Multi_Tap | 1.000 | 1.000 | 1.000 | 23 |
| MultiHand_Tap | 1.000 | 1.000 | 1.000 | 23 |
| Walk1 | 1.000 | 1.000 | 1.000 | 23 |
| Macro avg | 1.000 | 1.000 | 1.000 | 115 |
| Weighted avg | 1.000 | 1.000 | 1.000 | 115 |

### Confusion Matrix

Confusion Matrix — Predicted (columns) vs. True (rows)

|  | G_Tap | H_Tap | M_Tap | MH_Tap | Walk1 |
| --- | --- | --- | --- | --- | --- |
| Gentle_Tap | 23 | 0 | 0 | 0 | 0 |
| Hand_Tap | 0 | 23 | 0 | 0 | 0 |
| Multi_Tap | 0 | 0 | 23 | 0 | 0 |
| MultiHand_Tap | 0 | 0 | 0 | 23 | 0 |
| Walk1 | 0 | 0 | 0 | 0 | 23 |

The confusion matrix is a perfect 5×5 diagonal with zero misclassifications across all 115 test windows.

### SVM Comparison

For completeness, the SVM (RBF kernel, C=10, gamma=scale) achieved an accuracy of 0.9913. A single Hand_Tap window was misclassified as Multi_Tap; all other 114 windows were correctly classified.

### Feature Importance

Top 10 Features by Random Forest Gini Importance

| Rank | Feature | Importance |
| --- | --- | --- |
| 1 | AY_mad | 0.11763 |
| 2 | AX_mad | 0.07782 |
| 3 | AX_std | 0.06575 |
| 4 | AY_std | 0.06443 |
| 5 | AY_rms | 0.05378 |
| 6 | AZ_peak_to_peak | 0.04576 |
| 7 | AY_energy | 0.04535 |
| 8 | AY_min | 0.03870 |
| 9 | AX_zcr | 0.03528 |
| 10 | AZ_range_norm | 0.03444 |

### VibrationScore Analysis

VibrationScore Statistical Analysis

| Metric | Value |
| --- | --- |
| Non-zero samples | 5116 / 9989 (51.2%) |
| Zero samples | 48.8% |
| Min / Max | 0 / 29945 LSB |
| Mean | 1055.6 LSB |
| Std | 3297.7 LSB |
| Correlation with AX | 0.092 |
| Correlation with AY | −0.056 |
| Correlation with AZ | 0.366 |
| Correlation with dynamic_mag | 0.807 |

The strong correlation (r=0.807) with dynamic magnitude confirms that VibrationScore is a hand-crafted approximation of the same physical quantity already captured by dynamic_mag_rms. It therefore adds minimal new information and was excluded from the primary model.

### Model Artefacts Summary

Saved Model Artefact Summary

| Artefact | Value | Note |
| --- | --- | --- |
| Model name | RandomForest_AXYZonly | Best macro-F1 |
| Accuracy | 1.000 | On 115 test windows |
| Macro-F1 | 1.000 | Balanced across all 5 classes |
| Model file size | 311.3 KB | model.pkl (joblib) |
| Inference latency | 45.3 ms/window | Desktop Python benchmark |
| Window size | 50 samples (≈1.5 s) |  |
| Feature count | 40 | AX/AY/AZ stats + magnitude |
| Unit tests passing | 45 / 45 | All green |

## Test Suite Results

Unit Test Suite Results

| Module | Tests | Passed | Duration |
| --- | --- | --- | --- |
| test_preprocessing | 13 | 13 | – |
| test_features | 16 | 16 | – |
| test_inference | 16 | 16 | – |
| Total | 45 | 45 | 5.71 s |

## Challenges Faced

- Data leakage from overlapping windows: Naive random splitting causes windows from the same physical event to appear in both train and test sets. Resolved by a per-class temporal split (first 70% train, last 30% test).

- Irregular sampling intervals: Real hardware data exhibited timing jitter up to 292 ms between samples due to ESP32 scheduler preemption. Windows are defined in samples rather than wall-clock time to avoid sensitivity to this jitter.

- HTTP blocking on single core: Initial firmware ran the HTTP POST synchronously, blocking sensor acquisition for up to 8 seconds. Resolved by offloading the HTTP task to Core 0 via a FreeRTOS task.

- JSON payload size: Named-key JSON for 330 samples exceeded 20 KB causing ArduinoJson heap failures. The compact array format [[ax,ay,az,vs],...] reduced the payload to ≈6 KB.

- Limited real-world data: Only Gentle-Tap-R.csv contains genuine hardware recordings; the other four classes use synthetic simulations, limiting generalisability.

## Interpretation of Results

The perfect test accuracy (1.000) reflects that the five vibration classes are statistically well-separated in the 40-dimensional feature space. Mean absolute deviation (MAD) on the Y- and X-axes is the single most discriminative feature, consistent with vibration theory: MAD is less sensitive to outliers than standard deviation and directly captures average amplitude of deviation — a strong indicator of impact intensity and pattern type.

The domination of AY-axis features indicates that the sensor was mounted such that the Y-axis is most sensitive to the applied vibration direction, a hardware characteristic that would need re-evaluation if sensor orientation changes in deployment.

Important caveat: The 1.000 accuracy is a within-session result from a temporal split of single recordings. It confirms the feature set cleanly separates the five distributions as recorded, but does not guarantee cross-session or cross-operator generalisation. Real-world deployment requires independent recordings from multiple sessions per class.

# Conclusion

## Key Achievements

- Designed and implemented a complete end-to-end vibration classification system on ESP32 + MPU-6050 hardware, from raw sensor acquisition to real-time ML inference with networked output.

- Developed a 40-feature sliding-window extraction pipeline grounded in established vibration analysis theory (RMS, kurtosis, ZCR, MAD).

- Trained and evaluated three classifier variants, achieving a best accuracy and macro-F1 of 1.000 on 115 temporally-split held-out test windows.

- Eliminated data leakage through a per-class temporal split strategy with explicit documentation of the preliminary-evaluation caveat.

- Deployed a Flask REST API and browser dashboard enabling real-time inference for both the ESP32 firmware and any web client, with a dual-core FreeRTOS architecture ensuring uninterrupted sensor acquisition during HTTP transmission.

- Validated the complete system with a 45-test pytest suite (all passing in 5.71 seconds).

## What Was Learned

- Time-domain statistics such as MAD, kurtosis, and ZCR are highly effective for distinguishing mechanical vibration patterns at low sampling rates (∼33 Hz).

- JSON serialisation overhead, memory heap limits, and single-core blocking are practical IoT constraints requiring architectural solutions (FreeRTOS, compact payload formats).

- Proper temporal splitting, scaler fitting only on training data, and explicit labelling of preliminary results are essential practices for honest ML evaluation.

- The strong correlation between VibrationScore and dynamic magnitude (r=0.807) illustrates the principle of feature redundancy: hand-crafted features often recapitulate information already present in raw-axis statistics.

## Possible Improvements and Extensions

- Multi-session data collection: Record at least 5 independent sessions per class under different conditions to enable proper cross-session validation.

- On-device inference (TinyML): Convert the Random Forest to a TensorFlow Lite or Edge Impulse model to run directly on the ESP32, eliminating Wi-Fi round-trip latency.

- Frequency-domain features: Add FFT-based features (dominant frequency, spectral entropy, band energy ratios) to improve discrimination between periodic patterns.

- Probability calibration: Apply Platt scaling to calibrate RF posterior probabilities, making the confidence output statistically meaningful.

- Anomaly detection: Add an out-of-distribution detector (e.g., Isolation Forest) to flag events not belonging to any trained class, improving open-world robustness.

- Deep learning baseline: Evaluate a lightweight 1D-CNN or LSTM on raw window data once sufficient multi-session data is available.

- Continuous recording mode: Implement persistent on-device recording to store raw CSV data for all classes locally.

- OLED class probability bars: Extend the OLED display to show per-class probability bars for richer real-time feedback.

# References

99

M. Mirshekari, S. Pan, P. Zhang, Y. Erickson, and H. Noh, “Occupant Localization Using Footstep-Induced Structural Vibration,” Mechanical Systems and Signal Processing, vol. 112, pp. 77–97, 2016.

O. Avci, O. Abdeljaber, S. Kiranyaz, M. Hussein, M. Gabbouj, and D. Inman, “A review of vibration-based damage detection in civil structures: From traditional methods to Machine Learning and Deep Learning applications,” Mechanical Systems and Signal Processing, vol. 147, p. 107077, 2021.

O. D. Lara and M. A. Labrador, “A Survey on Human Activity Recognition using Wearable Sensors,” IEEE Communications Surveys & Tutorials, vol. 15, no. 3, pp. 1192–1209, 2013.

X. Guo, L. Chen, and C. Shen, “Hierarchical Adaptive Deep Convolution Neural Network and Its Application to Bearing Fault Diagnosis,” Measurement, vol. 93, pp. 490–502, 2016.

B. Sanchez-Lengeling, E. Reif, A. Pearce, and A. Wiltschko, “A Gentle Introduction to Graph Neural Networks,” Distill, 2021. [Online]. Available: https://distill.pub/2021/gnn-intro

L. Breiman, “Random Forests,” Machine Learning, vol. 45, pp. 5–32, 2001.

C. Cortes and V. Vapnik, “Support-vector networks,” Machine Learning, vol. 20, no. 3, pp. 273–297, 1995.

F. Pedregosa et al., “Scikit-learn: Machine Learning in Python,” Journal of Machine Learning Research, vol. 12, pp. 2825–2830, 2011.

InvenSense Inc., MPU-6000 and MPU-6050 Product Specification Revision 3.4, San Jose, CA, USA, 2013.

Espressif Systems, ESP32 Technical Reference Manual v5.0, Shanghai, China, 2023. [Online]. Available: https://www.espressif.com/sites/default/files/documentation/esp32_technical_reference_manual_en.pdf

International Organisation for Standardisation, ISO 10816-1: Mechanical Vibration — Evaluation of Machine Vibration by Measurements on Non-Rotating Parts, Geneva, Switzerland, 1995.

International Organisation for Standardisation, ISO 2631-1: Mechanical Vibration and Shock — Evaluation of Human Exposure to Whole-body Vibration, Geneva, Switzerland, 1997.

S. S. Rao, Mechanical Vibrations, 6th ed. Pearson Education, Hoboken, NJ, USA, 2017.